## Test 1: Normal Stability Eyes Open NSEO

Stability Score: 90.2%

Stability Class: Mildly Reduced

Pred. Direction of Sway: -87.8 deg

Fatigue Ratio: 116.0%

Adaptation Ratio: 0.0%

Directionality: 87%

Sway Center [Xo, Yo]: [0.0081 m, -0.0119 m]

ML Sway (Max & 95%): 0.0005 m & 0.0024 m

AP Sway (Max & 95%): 0.0148 m & 0.0175 m

Actual Sway (Max & Ave): 0.0110 m & 0.0039 m

Sway Velocity (Max & Ave): 0.015 m/s & 0.005 m/s

Sway Acceleration (Max & Ave): 0.426 m/s² & 0.099 m/s²

Sway Path Length: 0.0936 m

Area 95% Conf. Ellipse: 0.0000319 m²

LoS: 0.1796 m

Parameter p: 17.0

## Test 2: Normal Stability Eyes Closed NSEC

Stability Score: 81.9%

Stability Class: Severely Reduced

Pred. Direction of Sway: -85.7 deg

Fatigue Ratio: 0.0%

Adaptation Ratio: 21.6%

Directionality: 83%

Sway Center [Xo, Yo]: [0.0046 m, -0.0033 m]

ML Sway (Max & 95%): 0.0025 m & 0.0061 m

AP Sway (Max & 95%): 0.0300 m & 0.0325 m

Actual Sway (Max & Ave): 0.0205 m & 0.0072 m

Sway Velocity (Max & Ave): 0.048 m/s & 0.012 m/s

Sway Acceleration (Max & Ave): 0.836 m/s² & 0.181 m/s²

Sway Path Length: 0.2428 m

Area 95% Conf. Ellipse: 0.0001441 m²

LoS: 0.1796 m

Parameter p: 8.7

## Test 3: Perturbed Stability Eyes Open PSEO

Stability Score: 90.3%

Stability Class: Healthy Balance

Pred. Direction of Sway: 76.2 deg

Fatigue Ratio: 26.2%

Adaptation Ratio: 0.0%

Directionality: 15%

Sway Center [Xo, Yo]: [0.0095 m, -0.0151 m]

ML Sway (Max & 95%): 0.0031 m & 0.0150 m

AP Sway (Max & 95%): 0.0197 m & 0.0173 m

Actual Sway (Max & Ave): 0.0197 m & 0.0049 m

Sway Velocity (Max & Ave): 0.059 m/s & 0.017 m/s

Sway Acceleration (Max & Ave): 1.007 m/s² & 0.243 m/s²

Sway Path Length: 0.3357 m

Area 95% Conf. Ellipse: 0.0002036 m²

LoS: 0.1796 m

Parameter p: 4.2

## Test 4: Perturbed Stability Eyes Closed PSEC

Stability Score: 76.0%

Stability Class: Healthy Balance

Pred. Direction of Sway: -25.3 deg

Fatigue Ratio: 0.0%

Adaptation Ratio: 30.0%

Directionality: 25%

Sway Center [Xo, Yo]: [0.0074 m, 0.0035 m]

ML Sway (Max & 95%): 0.0070 m & 0.0413 m

AP Sway (Max & 95%): 0.0015 m & 0.0344 m

Actual Sway (Max & Ave): 0.0420 m & 0.0117 m

Sway Velocity (Max & Ave): 0.199 m/s & 0.042 m/s

Sway Acceleration (Max & Ave): 3.869 m/s² & 0.543 m/s²

Sway Path Length: 0.8322 m

Area 95% Conf. Ellipse: 0.0010879 m²

LoS: 0.1796 m

Parameter p: 3.7